FSAE Race Dashboard

Hardware & Software Technical Documentation

STM32 + Raspberry Pi Zero 2W Instrument Cluster

Version 1.0 — March 2026

Total System Cost: ~$90

# 1. System Overview

This document describes a custom digital instrument cluster designed for an FSAE formula-style race car. The system uses a two-processor architecture: an STM32F103RCT6 microcontroller handles real-time sensor acquisition, CAN bus communication, GPS, SD logging, and telemetry, while a Raspberry Pi Zero 2W runs a PyGame-based graphical dashboard with multiple configurable screen layouts.

## 1.1 Design Goals

- Sub-$120 total BOM cost for FSAE budget constraints

- Real-time data integrity: STM32 logs at 100Hz to SD, independent of display

- Driver-configurable screens with large, glanceable gauges (gear number primary)

- Wireless telemetry to pit crew via nRF24L01(planned)

- Hot-swap resilient: if Pi crashes, STM32 keeps logging

- Simulation mode for bench testing without the car

## 1.2 System Architecture

The system follows a strict separation of concerns:

| Layer | Hardware | Responsibility |
| --- | --- | --- |
| Real-Time | STM32F103RCT6 | CAN bus, GPS, analog sensors, SD logging at 100Hz, UART output, nRF24 telemetry |
| Display | Pi Zero 2W | PyGame dashboard, 9 screen layouts, settings UI, unit conversion |
| Communication | UART (115200) | CSV data at 25Hz from STM32 to Pi (10 fields per line) |
| Telemetry | nRF24L01+ | Wireless data relay to pit laptop (planned) |
| Storage | MicroSD on STM32 | Full-rate 100Hz logging with 17 columns per row |

# 2. Hardware Stack

## 2.1 Bill of Materials

| Component | Part | Est. Cost |
| --- | --- | --- |
| MCU | STM32F103RCT6 (Blue Pill+) | $9 |
| Display Computer | Raspberry Pi Zero 2W | $25 |
| Display | 7" 800x480 HDMI LCD | $17 |
| CAN Transceiver | SN65HVD230 module | $3 |
| GPS Module | u-blox NEO-6M / NEO-7M | $6 |
| SD Card Module | SPI breakout + 16GB card | $5 |
| Telemetry Radio | nRF24L01+ PA/LNA | $0 |
| Brake Pressure | 100 PSI analog sensor (5V) | $0 |
| Steering Angle | Rotary potentiometer | $0 |
| Voltage Regulator | 5V buck converter (12V in) | $3 |
| Connectors/Wire | Molex/JST, shielded CAN pair | $5 |
| Total |  | $75 |

## 2.2 STM32 Pin Assignments

| Function | Pins | Bus/Protocol |
| --- | --- | --- |
| CAN Bus | PA11 (RX), PA12 (TX) | CAN via SN65HVD230 |
| UART1 (to Pi) | PA9 (TX), PA10 (RX) | 115200 baud serial |
| UART2 (GPS) | PA2 (TX), PA3 (RX) | 9600 baud (NEO-6M default) |
| SPI1 (SD Card) | PA4 (CS), PA5 (SCK), PA6 (MISO), PA7 (MOSI) | SD FAT32 logging |
| SPI2 (nRF24) | PB12 (CS), PB13 (SCK), PB14 (MISO), PB15 (MOSI) | 2.4GHz telemetry |
| Analog In 0 | PA0 | Brake pressure sensor |
| Analog In 1 | PA1 | Steering angle pot |
| Sim Mode Jumper | PB0 | Pull LOW = simulation mode |
| Status LED | PC13 | Onboard LED (blink rate = mode) |

## 2.3 Wiring: STM32 to Pi

Only 3 wires connect the STM32 to the Pi Zero 2W:

| STM32 Pin | Pi GPIO | Function |
| --- | --- | --- |
| PA9 (UART1 TX) | GPIO15 (RX) | CSV data stream |
| GND | GND | Common ground |
| (PA10 RX) | (GPIO14 TX) | Optional: Pi-to-STM32 commands |

Important: Both devices run at 3.3V logic — no level shifter needed. The UART runs at 115200 baud. The Pi must have its serial console disabled (raspi-config) to free up /dev/ttyAMA0 for data.

## 2.4 CAN Bus Connection

The STM32F103 has a built-in CAN peripheral. An SN65HVD230 transceiver converts between the STM32's logic-level CAN and the differential CAN bus on the car. The Speeduino Dropbear v2 ECU broadcasts engine data on CAN IDs 0x5F0–0x5F3 which contain RPM, coolant temp, TPS, MAP, AFR, ignition advance, and more.

A 120Ω termination resistor should be placed at each end of the CAN bus (one at the ECU, one at the STM32 transceiver).

## 2.5 GPS Module

The u-blox NEO-6M (or NEO-7M) module connects to UART2 at 9600 baud by default. It outputs NMEA sentences which are parsed by the TinyGPSPlus library running on the STM32. Parsed fields include latitude, longitude, speed over ground, altitude, fix quality, and satellite count. The module needs a clear view of the sky — mount the ceramic patch antenna on top of the chassis with the active side facing up.

## 2.6 SD Card Logging

The STM32 logs all sensor data to a MicroSD card via SPI1 at 100Hz. The SD card must be formatted FAT32. Log files are created automatically as LOG_001.csv, LOG_002.csv, etc. The buffer is flushed every 1 second to prevent data loss on power cut. Each row contains 17 fields:

TIMESTAMP_MS, RPM, SPEED_MPH, THROTTLE_PCT, BRAKE_PCT, CLT_F, OIL_PSI, AFR, VOLTAGE, LAT, LON, GPS_SPEED_MPH, GPS_ALT_FT, GPS_SATS, GPS_FIX, AN0, AN1

This runs on SPI1, which is a separate bus from the nRF24 telemetry on SPI2, so both can operate simultaneously without contention.

## 2.7 Simulation Mode

Pulling pin PB0 LOW at boot activates simulation mode. The STM32 generates fake sweeping data (RPM sweeps 1000–13500, speed/throttle/brake cycle, temperatures oscillate) and sends it over UART exactly as if live sensors were connected. The onboard LED blinks at 500ms intervals in sim mode vs 2000ms in live mode. This allows full bench testing of the display without the car.

# 3. Communication Protocol

## 3.1 CSV over UART

The STM32 sends a simple CSV line to the Pi at 25Hz over UART1 at 115200 baud. Each line is newline-terminated and contains exactly 10 fields:

RPM,SPEED,THROTTLE,BRAKE,CLT,OIL,LAT,LON,GPS_SPD,GPS_SATS

| Field | Description |
| --- | --- |
| RPM | Engine RPM (integer) |
| SPEED | Wheel speed in MPH (integer) |
| THROTTLE | Throttle position 0–100% |
| BRAKE | Brake pressure 0–100% |
| CLT | Coolant temperature in °F |
| OIL | Oil pressure in PSI |
| LAT | GPS latitude (decimal degrees) |
| LON | GPS longitude (decimal degrees) |
| GPS_SPD | GPS speed over ground (MPH) |
| GPS_SATS | Number of GPS satellites locked |

Lines starting with # are comment/status messages from the STM32 (e.g., boot status, SD card errors). The Pi's parser silently discards these. All values are transmitted in imperial units (°F, MPH, PSI) and converted at display time based on the driver's unit preferences.

## 3.2 Why CSV Instead of Binary

- Human-readable: plug in a USB-serial adapter and see data in any terminal

- Easy to debug: no byte-alignment issues, no endianness concerns

- Minimal overhead: 10 fields at 25Hz is roughly 2KB/s — trivial for 115200 baud

- Same parser for live and simulation modes

# 4. Software: STM32 Firmware

## 4.1 Build System

The firmware is built with PlatformIO using the Arduino framework for STM32. The target board is genericSTM32F103RC (64KB RAM, 256KB flash). Upload via ST-Link V2 programmer or USB DFU bootloader.

Platform: ststm32

Framework: arduino

Board: genericSTM32F103RC

Upload: ST-Link V2 (SWD) or USB DFU

Monitor Baud: 115200

## 4.2 Main Loop (100Hz)

The main loop runs at 100Hz (10ms interval) and performs the following on each tick:

- Read CAN bus — decode Speeduino frames 0x5F0–0x5F3 into RPM, CLT, TPS, MAP, AFR, etc.

- Read analog inputs — PA0 (brake pressure), PA1 (steering angle)

- Parse GPS — feed UART2 bytes to TinyGPSPlus, extract lat/lon/speed/sats

- Log to SD — write 17-column CSV row at full 100Hz rate

- Send to Pi — every 4th tick (25Hz), send 10-field CSV line over UART1

- Flush SD — every 1 second, flush the file buffer to prevent data loss

## 4.3 Key Libraries

| Library | Purpose |
| --- | --- |
| TinyGPSPlus | NMEA sentence parsing for u-blox GPS |
| SdFat / SD | FAT32 SD card access via SPI1 |
| RF24 (planned) | nRF24L01+ communication for pit telemetry |
| eXoCAN / STM32_CAN (planned) | Native CAN peripheral driver for STM32F103 |

## 4.4 Implementation Status

| Feature | Status |
| --- | --- |
| UART CSV output to Pi | Complete — 10 fields at 25Hz |
| GPS parsing (TinyGPSPlus) | Complete — lat/lon/speed/sats |
| SD card logging (100Hz) | Complete — 17 columns, auto-numbered files |
| Simulation mode (PB0 jumper) | Complete — fake sweeping data for bench testing |
| Analog inputs (brake/steering) | Complete — PA0/PA1 ADC reads |
| CAN bus init + Speeduino decode | TODO — use eXoCAN, parse 0x5F0–0x5F3 |
| nRF24 telemetry to pit | TODO — RF24 library on SPI2 |

# 5. Software: Pi Dashboard

## 5.1 Architecture

The Pi-side software consists of three Python files:

| File | Purpose |
| --- | --- |
| race_dash_config.py | Centralized configuration, unit conversion, color themes, settings definitions |
| race_dash_core.py | UART thread, CSV parsing, signal buffer, simulation fallback |
| race_dash_pygame.py | PyGame rendering, 9 screen classes, settings UI, screen toggle system |

The UARTThread runs in a background thread, reads CSV lines from /dev/ttyAMA0, parses them, and writes values into a thread-safe SignalBuffer. The main PyGame loop reads from the buffer at the configured FPS (default 30) and draws the active screen.

## 5.2 Screen System

There are 9 available screens plus a Settings screen. Drivers can enable or disable screens from Settings > Screens. Swiping left/right (touchscreen) or arrow keys cycle through enabled screens only. Each screen is a Python class with a NAME, DESC, and draw() method.

| Screen | ID | Description |
| --- | --- | --- |
| Lap Timer | lap_timer | Giant gear center, shift lights, lap times, speed, temps |
| Main Gauge | main_gauge | Gear inside RPM arc gauge with shift lights |
| Drag | drag | Giant speed + gear, minimal clutter for acceleration runs |
| Driver Strip | strip | Horizontal bar layout — RPM bar, speed, gear, throttle/brake |
| GPS | gps | Coordinates, satellite count, speed trace graph |
| Diagnostics | diagnostics | 9-cell grid of all sensor values |
| Warnings | warnings | Full-screen warning panels or all-clear status |
| C4 Corvette | c4_corvette | 80s digital dash tribute with segmented arc gauges |
| Classic | classic | Traditional analog round gauges with sweep needles |

## 5.3 Adding a New Screen

To add a new screen, create a class with NAME, DESC, and a draw(surface, data, fonts, page_idx, page_total) method, then add one line to SCREEN_REGISTRY. It will automatically appear in the Settings > Screens toggle list.

## 5.4 Configuration System

All settings are stored in config.json and editable from the on-screen Settings page (press 'S' or swipe to the last page). The config system supports:

- Unit preferences: imperial (°F, MPH, PSI) or metric (°C, km/h, bar)

- Engine parameters: redline RPM, shift point, max RPM, gear speed thresholds

- Warning thresholds: coolant high/critical, oil low/critical

- Display settings: FPS, brightness, screen rotation

- Driver/event info: driver name, event name

- Screen selection: which of the 9 screens are enabled per driver

## 5.5 Key Design Decisions

- Gear number is the most prominent element on racing screens (size 220-240, dead center)

- Shift lights are thick (54px) with center-out activation pattern

- All units stored as imperial internally, converted only at display time

- CSV parsing preserves raw data integrity — buffer values are never modified

- Screen instances are created once and reused, never recreated on toggle

- Simulation mode builds CSV strings and parses through the same code path as live data

# 6. Deployment

## 6.1 Flashing the STM32

Install PlatformIO (VS Code extension or CLI). From the stm32_firmware directory:

pio run -t upload

For bench testing without the car, connect a jumper wire from PB0 to GND before powering on. The LED will blink at 500ms to confirm simulation mode is active.

## 6.2 Setting Up the Pi

- Install Raspberry Pi OS Lite (no desktop needed)

- Disable serial console: sudo raspi-config > Interface > Serial > No console, Yes hardware

- Install dependencies: sudo apt install python3-pygame python3-serial

- Copy race_dash_config.py, race_dash_core.py, race_dash_pygame.py to /home/pi/dash/

- Wire STM32 PA9 (TX) to Pi GPIO15 (RX), and GND to GND

- Run: python3 race_dash_pygame.py

## 6.3 Auto-Start on Boot

To start the dashboard automatically when the Pi powers on, add a systemd service or add the following to /etc/rc.local before the 'exit 0' line:

cd /home/pi/dash && python3 race_dash_pygame.py &

## 6.4 Controls

| Input | Action |
| --- | --- |
| Swipe left / Right arrow / Space | Next screen |
| Swipe right / Left arrow | Previous screen |
| Press 'S' | Jump to Settings |
| Tab (in Settings) | Cycle settings tabs |
| Up/Down arrows (in Settings) | Navigate rows |
| Enter/Backspace (in Settings) | Adjust values |
| Escape | Quit |

# 7. Future Work

- Implement CAN bus initialization using eXoCAN library and decode Speeduino CAN IDs 0x5F0–0x5F3

- Implement nRF24L01 telemetry: STM32 broadcasts sensor data to a pit-side receiver

- Add lap timing via GPS geofence or IR beacon receiver

- Driver profile export/import (save config.json per driver)

- Optional: Pi sends driver name and event info back to STM32 over UART for SD log file headers

- Data analysis tool: Python script to parse SD log CSVs and generate lap-by-lap performance reports